Matrix multiplication relation with solving Linear equations :

Matrix multiplication has been defined in such a way that it compactly represents a system of linear equations .

In matrix representation , we can write it as multiplication of two matrices .

=> AX = B => A^ (-1) AX = A^(-1)B => X = A^(-1) B .

So if A is invertible and  we can find A^(-1) , then we can get the values x1, x2, ……, xn , i.e., X matrix by multiplying A^(-1) & B .

Kinematics VS Inverse Kinematics .

Kinematics à study of motion (without considering forces or torques )

- Tells us cover the robot’s end effects will be (x1,x2) if we know the joint angles ( Θ1, Θ2)

Inverse Kinematics à we know the desired position of the end effector (x,y) and we must find the joint angles ( Θ1, Θ2) that will reach that point .

Eg :- initial configuration

Let +ve  Θ be the angle in the clockwise disection wrt to OE .

If  Θ = 270 , then the final configuration will be ON .

This is Kinematics .

Now , if we were only the final configuration to be ON, and were told to find  Θ , then  Θ = (2n π  + 3 π /2) , where n = 0,1,2,……

And also  Θ = - π /2 + 2n π  ; n = 0,1,2,…….

So we have more than one solution.

This is inverse Kinematics.

(2). Example of 2-arm , 2-joint Robotic arm.

Kinematics.

Given  Θ1 = 30° and  Θ2 = 60°, find the final position of the end effector (x,y).

X = l1cos Θ1 + l2cos( Θ1, Θ2)

= l1cos30° + l2cos90°

= (3)(-1)/2 l1

Y = l1sin Θ1 + l2sin( Θ1, Θ2)

= l1sin30° + l2sin90° = l1/2 + l2

Inverse Kinematics .

Given x , y = ((2-√2)/2√2 , (2-√6)/2√2 ). Find the corresponding  Θ1 ,  Θ2

Consider l1 = l2 = 1

:. X = (2-√2)/2√2 = 1/√2 – ½

- Cos Θ1 + cos( Θ1, Θ2) = 1/√2 – ½

:. Cos Θ1 = 1/√2 , cos( Θ1, Θ2) =  - ½

Θ1 lies in 1st quadrant & Θ1 + Θ2 lies in 3rd quadrant.

Θ1 = 45° and Θ1+ Θ2 = 240°

:. Θ1 = 45° & Θ2 = 195°

Manipulators

- Mechanical devices designed to interact with the environment by handling grasping and manipulating objects.

- Combination of links and joints.

- Manipulators are made of actuators that are devices that produce motion (linear/rotators) by converting a control signal and an energy source into mechanical output. (eg:- DC , servo & stepper motors).

Work space/ configuration space / Task space

- Area or volume where robot can actually work.

- Set of all points that the end – effects can reach , either position or orientationalls , given the robots joint limits and link lengths.

Joints space

Max region the joint can cover.

Degree of freedom

- In robotics, it is the number of independent motors (in general , actuators)

Safe limit :-> maximum allowable operating boundaries beyond which a robotic motion , force or speed could become unsafe for itself , its surroundings . (eg :- speed limit , force limit , joint torque limiters )

- Prevents mechanical damage.

Inverse Kinematics for a 2 – dofs Manipulator

L1cosQ1 + l2cos(Q1+Q2) = X -- (1)

L1sinQ1 + l2sin(Q1+Q2) = Y -- (2)

Now (1)^2 + (2) ^2 , yields

X^2 + Y^2 = (l1)^2 + (l2)^2 + 2(l1)(l2) [ cosQ1 cos(Q1 + Q2)  +  sinQ1 sin(Q1+ Q2)  ]